# LA1.16i - AMPLIFIED CONTROLLER FOR INSTALL

System control

# LA1.16i

## Short description

Amplifier with integrated DSP controller – 16 x 80 at 8 ohms / 16 x 160 W at 4 ohms.

## Description

System amplifier (class D) for installations, with an output power of 16 x 160 Watts at 4 ohms, A/D converter, and integrated DSP processor. Flexible 16 x 16 routing and summation matrix with 16 inputs and 16 outputs. Power supply via dual SMPS (Switched Mode Power Supply) with power factor correction (PFC) and 24V DC backup supply input. Advanced DSP algorithms optimize real-time performance from available SMPS and amplifier resources. Heat dissipation by DSP-controlled fans. Analog audio input and digital (AES/EBU) audio input with active links. Phoenix terminals for input, output, and GPIO connections. Ethernet connections to pass network audio and for remote control and monitoring. The DSPs process IIR and linear phase FIR filters enable real-time adjustments, contour filters, and filters to compensate for air absorption. Protection against thermal overload and over-excursion. Built-in library with factory presets for all manufacturers' loudspeakers and 10 user memories.

Ethernet-based control network with up to 1 Gbit/s. Support of digital audio signal transmission via the Milan-AVB and AES67 protocols with seamless redundancy. "Automatic fallback" options are also possible. Design of control networks with up to 253 controller power amplifiers in star, daisy chain, or hybrid arrangement. The controller power amplifiers and computers in the network are connected via CAT5e U/FTP or higher, with standard RJ45 connectors. The network management software can be run on both Windows and Mac OS X and is designed to include component load, signal levels, limiter activity, overload, and fault messages. It is also used to manage presets, as well as to adjust input gain, mute, delay correction, and filtering. Integration into higher-level media control systems is possible by means of software modules.

## Technical requirements

Amplification:

- Output power (12 dB Crest Factor 2 ms sine bursts, 1 kHz, < 1% THD, all channels):

  - 16 x 80 W at 8 ohms (Single Ended mode)

  - 16 x 160 W at 4 ohms (Single Ended mode)

  - 8 x 300 W at 8 ohms (Bridged Tied Load mode)

- Amplification class: High-efficiency Class D

- Architecture: 16 x 16

- Frequency range:  20 Hz - 20 kHz (+/- 0.25 dB)

- Distortion THD+N: < 0.003% (at 1 kHz, 8 ohms, unweighted)

- Output dynamic range: > 114 dB (20 Hz - 20 kHz, 8 ohms, A-weighted)

- Noise level < -85 dBV (20 Hz - 20 kHz, 8 ohms, A-weighted)

- Amplification gain: 26 dB (SE), 32 dB (BTL)

- Channel separation: > 90 dB (1 kHz, 8 ohms)

- Output delay: adjustable from 0 to 1000 ms

DSP:

- Gen. 5 Dual SHARC 32-bit, floating-point, 96 kHz Sampling frequency

- Routing inputs/outputs: flexible 16 x 16 routing and summation matrix

- Latency for analog and digital inputs:

  - In standard operating mode: 3.84 ms

  - In low latency operating mode: 1.18 ms

AVB input mode

- 128 channels 48 kHz / 96 kHz from 16 streams of up to 8 channels

- Standards: Ethernet AVB IEEE 802.1BA-2011, Number: 1, Class: A

- Latency: 2 ms max

- Formats: AAF PCM32 or IEC 61883-6 AM824 at 48 kHz or 96 kHz

- Clock: synchronized on clock of the connected AVB input stream

- MILAN™- certified

AES67 input mode

- 128 channels 48 kHz from 16 streams of up to 8 channels

- Standards: AES67-2023 Transport: RTP over UDP/IP

- Latency: Packet times supported: 0.333 ms or 1 ms, default latency: 3 packet times

- Formats: L16, up to 8 channels, at 48 kHz, L24, up to 8 channels, at 48 kHz

- Clock: IEEE 1588-2008, PTPv2

Analog input:

- 1 channel with active link and failsafe relay

- Input impedance: 22 kOhm (balanced)

- Maximum input level: 22 dBu (balanced, THD 1%)

- 32-bit A/D converter (121 dB dynamic range, 20 Hz - 20 kHz, A-weighted)

Digital input:

- 2 channels with active link and failsafe relay

- Standards: AES/EBU (AES3)

- Supported Sampling frequency: 44.1 kHz to 192 kHz

- Word length: 16 to 24 bits

Sample Rate Converter (SRC)

- Sampling frequency: 96 kHz

- Word length: 24 bits

- Dynamic range: 140 dB

- Distortion THD+N: < -120 dBFS

Input gain:

- Range: -12 dB to + 12 dB

- Steps: 0.1 dB

Connectors:

- Dual-port Ethernet Gigabit interface with 2 x RJ45/etherCONTM1 Gb/s

- 12-point terminal block utilizing:

  - Inputs: 3-points for AES/EBU or analog

  - Input links: 3-points for AES/EBU or analog

  - General Purpose I/O (GPIO): 3 GPIO, isolated optocoupler inputs, isolated relay contacts 4-points

  - Backup power circuit for the DSP: 24 V DC 2-point terminal block

- Mains rating: 100 V AC - 240 V AC +/- 10%, 50 Hz - 60 Hz

- Nominal current requirements: 10 A for 200 V AC - 240 V AC

- Loudspeaker outputs: 8 female x 4-point terminal blocks

Display:

- Information LED: power, network, status

- Input channels LED: signal, limit, clip

- Output channels LED: signal, limit, clip

Third-party management solutions:

- QSC Q-SYS™

- Crestron®

- HTTP

Operating conditions:

- Operating temperature: between -5 °C / 23 °F and 50 °C / 122 °F

- Cooling system: DSP-controlled fans

Physical data:

- Dimensions (W, H, D): 483 mm, 88 mm, 434 mm / 19 in, 3.5 in, 17.1 in

- Rack mounting: 1U

- Weight (net): 5.6 kg / 12.3 lb

- IP: IP2x

- Finish: Black

Product name: LA1.16i

Manufacturer: L-Acoustics