108P SYSTEM

- The system shall consist of a bi-amplified 2-way active enclosure. The high frequency transducer shall feature a 1.5-in diaphragm compression driver coaxially-loaded onto an 8-in weather resistant low frequency transducer in a bass-reflex tuned enclosure.

The usable system bandwidth shall be 55 Hz to 20 kHz (-10 dB). The system shall generate a conical directivity (-6 dB points) of 100°. Maximum peak SPL, measured at 1m under free field conditions using pink noise with crest factor 4, shall be 127 dB.

- Enclosure cabinet construction shall consist of Baltic birch plywood. Dimensions shall be 421 mm (16.6 in) height, 250 mm (9.8 in) width and 299 mm (11.8 in) depth. Enclosure weight shall be 13 kg (28.6 lbs). The front of the enclosure shall be protected by a sandblast-coated steel grill covered with acoustically-transparent fabric. The enclosure shall have a wedge-shaped design with a fixed angle setting of 30° for monitoring applications.

The loudspeaker shall feature a 35 mm (1.38 in) diameter pole mount socket mounted on the bottom side, and a recessed plate on the top side, for rigging the enclosure in conjunction with an adjustable U-bracket assembly. Connection to the loudspeaker shall be made via two parallel XLR connectors. AC power connectors shall be PowerCon® type and 115V or 230V operation shall be selectable via rear panel slider switch.

- The system contains a speaker management DSP and a 2-channel power amplification module. The digital filter manages the system crossover network, driver time alignment, thermal protection and the linearization of the transducers.

The input shall be electronically balanced and the max input level shall be +12 dBu (gain potentiometer at 0 dB position). The power amplifier shall provide 500 W continuous into 4 ohms for the low section and 250 W continuous into 8 ohms for the high section. DSP sampling shall be 24 bits/ 48kHz.

- The loudspeaker system shall be the L-ACOUSTICS® 108P.

- The U-Bracket shall be the L-ACOUSTICS® ETR8-2.

The response curves of the system are available upon request from applications@l-acoustics.com.