5XT - Short throw point source

X Series

Short throw: 15 m

Single element deployment (horizontal or vertical)

Axisymmetric directivity

# 5XT

## Short description

2-way passive enclosure with 5 in / 1 in speaker producing an axisymmetric point source directivity of 110 degrees with consistent off-axis response.

## Description

2-way passive enclosure with a premium 1 in diaphragm compression driver coaxially loaded by a 5 in low-mid frequency transducer mounted in a bass-reflex cabinet. The enclosure cabinet is constructed of premium grade Baltic birch plywood and can be custom colored based on the RAL color program. The enclosure front is protected by a coated steel grill. The compact enclosure has been designed to integrate in any situation. The dedicated bracket ensures wall-mount and ceiling-mount configurations. A 3/8 in insert allows for pole-mount configurations. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Nominal directivity: 110° axisymmetric

Transducers:

- LF transducer: 5" cone driver

- HF transducer: 1" diaphragm compression driver

Acoustics:

- Nominal impedance: 16 Ω

- Usable bandwidth: 95 Hz - 20 kHz (-10 dB), 105 Hz - 20 kHz (-6 dB), 115 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 66 W

- Maximum SPL: 121 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 165 mm, 165 mm, 165 mm / 6.5 in, 6.5 in, 6.5 in

- Weight (net): 3.5 kg / 7.7 lb

- Connectors: 2 × 4-point speakON

- IP: IP30 / IP54 with optional connector sealing plate and a down-tilt angle of 20° and more

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: 5XT

Manufacturer: L-Acoustics