K3i - Long throw line source

K Series Install

Long throw > 35 m

Deployment in vertical lines

Incremental coverage with variable inter-element angles

# K3i

## Short description

Install-specific 2-way active enclosure with two 12 in drivers and one 4 in diaphragm compression driver - 10-degree enclosure angle with adjustable waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

Install-specific 2-way active enclosure with two weather-resistant premium 12 in transducers and one high-output 4 in diaphragm compression driver coupled to a cylindrical waveguide with user-adjustable horizontal directivity and 10° of vertical coverage. The enclosure cabinet is constructed of premium grade Baltic birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates inserts for flush-fitting connecting plates and an optional acoustically transparent front screen. The four-point hardware allows for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source vertical directivity depends upon the number of elements and inter-element angles. Horizontal directivity can be adjusted to four settings to match any audience geometry: 110°, 70° and 90° asymmetric on either side. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: depending on number of elements and array curvature

- Waveguide directivity: 70° / 110° symmetric or 90° asymmetric

Transducers:

- LF transducer: 12" cone driver

- HF transducer: 4" diaphragm compression driver

Acoustics:

- Nominal impedance:  8 Ω LF, 8 Ω HF

- Usable bandwidth: 42 Hz - 20 kHz (-10 dB), 46 Hz - 20 kHz (-6 dB), 50 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth):

  - LF:  700 W

  - HF:  130 W

- Maximum SPL: 143 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 907 mm, 357 mm, 429 mm / 35.7 in, 14.1 in, 16.9 in

- Weight (net): 35 kg / 77.2 lb

- Connectors: 2 × 4-point terminal blocks with push-in connection

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: K3i

Manufacturer: L-Acoustics