Kiva II - Long throw line source

K Series

Long throw > 35 m

Deployment in vertical lines

Incremental coverage with variable inter-element angles

# Kiva II

## Short description

2-way passive colinear source with two 6.5 in drivers and one 1.75 in diaphragm compression driver - 15-degree enclosure angle with 100° horizontal waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

2-way passive enclosure with two weather-resistant premium 6.5 in transducers and high output 1.75 in diaphragm compression driver coupled to a cylindrical waveguide with 110 degrees of horizontal coverage and 15° of vertical coverage. The enclosure cabinet is constructed of moisture-resistant composite material. The enclosure front is protected by a composite grill and an acoustically transparent fabric. The enclosure integrates two handles. A three-point captive hardware allows for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source directivity depends upon the number of elements and inter-element angles and can be matched to the audience geometry. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Enclosure directivity: 15° (single), depending on the number of elements and array curvature

- Waveguide directivity: 100° symmetrical

Transducers:

- LF transducer: 6.5" cone driver

- HF transducer: 1.75" diaphragm compression driver

Acoustics:

- Nominal impedance: 16 Ω

- Usable bandwidth: 70 Hz - 20 kHz (-10 dB), 75 Hz - 20 kHz (-6 dB), 84 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 186 W

- Maximum SPL: 138 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: composite sandwich structure

- Dimensions (W, H, D): 525 mm, 202 mm, 357 mm / 20.7 in, 8 in, 14 in

- Weight (net): 14 kg / 31 lb

- Connectors: 2 × 4-point speakON

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: Kiva II

Manufacturer: L-Acoustics