L2 - Long throw PROGRESSIVE line source

L Series

Long throw > 35 m

Deployment in vertical lines

Progressive 10 degree vertical coverage

# L2

## Short description

16-channel active element with integrated cardioid and optimized mechanical arrangement. Using four 12 in drivers, eight 10 in drivers and eight 3 in diaphragm compression drivers - 10-degree progressive enclosure angle with adjustable horizontal waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

16-channel active element, divided into four modules, with a total of 10° of optimized progressive mechanical vertical coverage and integrated cardioid. Using four side-mounted 12 in transducers for low frequency cardioid, eight front facing 10 in transducers for low frequencies, and eight high-output 3 in diaphragm compression drivers coupled to a cylindrical waveguide with user-adjustable horizontal directivity per module. The enclosure cabinet is constructed of premium grade Baltic birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates four ergonomic handles and four ground runners. Flush, four-point captive, auto-locking, hardware allows rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source vertical directivity depends upon the number and type of elements. Horizontal directivity can be adjusted to four settings to match any audience geometry: 110°, 70° and 90° asymmetric on either side. The integrated cardioid can be selected between cardioid or supercardioid patterns. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: 10° Full line directivity depends on the number of elements

- Waveguide directivity: 70° / 110° symmetric or 90° asymmetric

- Active Cardioid: Cardioid or Supercardioid

Transducers:

- LC transducer: 12” cone driver

- LF transducer: 10" cone driver

- HF transducer: 3" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω LC, 8 Ω LF, 8 Ω HF (16 channels of 8 Ω each)

- Usable bandwidth: 45 Hz - 20 kHz (-10 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth):

  - LC: 561 W

  - LF: 644 W

  - HF: 381 W

- Maximum SPL: (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

  - 155 dB for the full element

  - 147 dB for the first module

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 850 mm, 1197 mm, 559 mm / 33.5 in, 47.2 in, 22 in

- Weight (net): 158 kg / 348 lb

- Connector: 1 × 37-point (32 points used) connector

- IP: IP55

- Finish: dark grey brown Pantone 426 C

Product name: L2

Manufacturer: L-Acoustics