# L2D - Long throw PROGRESSIVE line source

L Series

Long throw > 35 m

Deployment in vertical lines

Progressive 60 degree vertical coverage

# L2D

## Short description

16-channel active element with integrated cardioid and optimized mechanical arrangement. Using four 12 in drivers, eight 10 in drivers and eight 3 in diaphragm compression drivers - 60-degree progressive enclosure angle with adjustable horizontal waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

16-channel active element, divided into four modules, with a total of 10° of optimized progressive mechanical vertical coverage and integrated cardioid. Using four side-mounted 12 in transducers for low frequency cardioid, eight front facing 10 in transducers for low frequencies, and eight high-output 3 in diaphragm compression drivers coupled to a cylindrical waveguide with user-adjustable horizontal directivity for the upper two modules. The enclosure cabinet is constructed of premium grade Baltic birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric and integrates four ergonomic handles. Flush, four-point captive, auto-locking, hardware allows rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source vertical directivity depends upon the number and type of elements. Horizontal directivity of the upper two modules can be adjusted to four settings to match any audience geometry: 110°, 70° and 90° asymmetric on either side. The lower two modules have a progressive horizontal directivity from 110° to 140°. The integrated cardioid can be selected between cardioid or supercardioid patterns. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: 60°; Full line directivity depends on the number of elements

- Waveguide directivity: upper modules 70° / 110° symmetric or 90° asymmetric, lower modules fixed progressive from 110° to 140°

- Active Cardioid: Cardioid or Supercardioid

Transducers:

- LC transducer: 12” cone driver

- LF transducer: 10" cone driver

- HF transducer: 3" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω LC, 8 Ω LF, 8 Ω HF (16 channels of 8 Ω each)

- Usable bandwidth: 45 Hz - 20 kHz (-10 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth):

  - LC: 561 W

  - LF: 644 W

  - HF: 381 W

- Maximum SPL: (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

  - 151 dB for the full element

  - 147 dB for the first module

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 850 mm, 1252 mm, 559 mm / 33.5 in, 49.3 in, 22 in

- Weight (net): 149 kg / 328 lb

- Connector: 1 × 37-point (32 points used) connector

- IP: IP55

- Finish: dark grey brown Pantone 426 C

Product name: L2D

Manufacturer: L-Acoustics