SB10i - Subwoofer

Install-specific 10' Subwoofer

Mounted on-wall, on-ceiling, in-wall, in-ceiling or stacked on the ground

# SB10i

## Short description

Install-specific ultra-compact subwoofer featuring a single 10 in with optimized laminar vents to maximize LF efficiency - deployed in stacked or wall-mounted configurations.

## Description

Install-specific ultra-compact passive subwoofer with a direct radiating 10 in transducer mounted in a bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The transducer front is protected by a coated steel grill and an acoustically transparent fabric. Dedicated accessories allow for various mounting options: on-wall, on-ceiling, in-wall and in-ceiling. The inserts and the connectors are all gathered on a single enclosure panel. Rubber feet stabilize ground-stacked deployment of one or more elements to prevent movement or cabinet scratching. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: standard

Transducers:

- LF transducer: 10" cone driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth:

  - Preset 1: 25 – 70 Hz (-10 dB), 27 – 64 Hz (-6 dB), 30 – 55 Hz (-3 dB)

  - Preset 2: 27 – 118 Hz (-10 dB), 30 – 96 Hz (-6 dB), 33 – 80 Hz (-3 dB)

  - Preset 3: 29 - 180 Hz (-10 dB), 32 - 125 Hz (-6 dB), 35 - 87 Hz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 146 W

- Maximum SPL (Peak level at 1 m under half space conditions using pink noise with crest factor 4):

  - Preset 1: 119 dB

  - Preset 2: 122 dB

  - Preset 3: 124 dB

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 540 mm, 540 mm, 170 mm / 21.3 in, 21.3 in, 16.6 in

- Weight (net): 14 kg / 31 lb

- Connectors: 1 × 4-point terminal block with push-in connection

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: SB10i

Manufacturer: L-Acoustics