SB18 IIi - Subwoofer

Install-specific 18' Subwoofer

Flown or stacked deployment

Omni or cardioid configurations

# SB18 IIi

## Short description

Install-specific high-power subwoofer featuring a single 18 in speaker with high excursion driver and optimized laminar vents to maximize LF efficiency - deployed in omnidirectional or cardioid patterns in stacked or flown configurations.

## Description

Install-specific passive subwoofer with a high excursion, direct radiating 18 in transducer mounted in a dual bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The transducer front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates inserts for flush-fitting connecting plates and an optional acoustically transparent front screen. The four-point hardware allows for rigging and combining with other loudspeakers of the same family. Dedicated rigging accessories allow for flown or stacked configurations. Combining two or more elements with associated presets achieve omnidirectional or cardioid directivity patterns. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: standard or cardioid configuration

Transducers:

- LF transducer: 18" cone driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 32 Hz - 110 Hz (-10 dB), 34 Hz - 100 Hz (-6 dB), 37 Hz - 82 Hz (-3 dB)  / 32 Hz - 80 Hz (-10 dB), 32 Hz - 70 Hz (-6 dB), 35 Hz - 60 Hz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 387 W

- Maximum SPL: 138 dB Peak level at 1 m under half space conditions using pink noise with crest factor 4

Physical data:

- Acoustical load: dual bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 701 mm, 540 mm, 728 mm / 27,6 in, 21.3 in, 28.7 in

- Weight (net): 48 kg / 106 lb

- Connectors: 1 × 4-point terminal block with push-in connection

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: SB18 IIi

Manufacturer: L-Acoustics