SB18 - Subwoofer

18' Subwoofer

Flown or stacked deployment

Omni or cardioid configurations

# SB18

## Short description

Compact subwoofer featuring a single 18 in speaker with high excursion drivers and optimized laminar vents to maximize LF efficiency - deployed in omnidirectional or cardioid patterns in stacked or flown configurations.

## Description

Compact passive subwoofer with a high-excursion, direct-radiating 18 in transducer mounted in a dual bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates four ergonomic handles, and panel corner stops for enclosure protection. The captive hardware allows for rigging and combining with other loudspeakers of the same family. Dedicated rigging accessories allow for flown or stacked configurations. Combining two or more elements with associated presets achieve omnidirectional or cardioid directivity patterns. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: standard or cardioid

Transducers:

- LF transducer: 18"

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 32 Hz - 110 Hz (-10 dB), 34 Hz - 100 Hz (-6 dB), 37 Hz - 82 Hz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 387 W

- Maximum SPL: 138 dB Peak level at 1 m under half space conditions using pink noise with crest factor 4

Physical data:

- Acoustical load: dual bass-reflex

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 750 mm, 553 mm, 707 mm / 29.5 in, 21.8 in, 27.8 in

- Weight (net): 52 kg / 115 lb

- Connectors: IN: 1 × 4-point speakON/ LINK:  1 × 4-point speakON

- Finish: dark grey brown Pantone 426 C

Product name: SB18

Manufacturer: L-Acoustics