SB6i - Subwoofer

Ultra-shallow install-specific 6.5' Subwoofer

Mounted on-wall, on-ceiling, in-wall and in-ceiling

# SB6i

## Short description

Install-specific ultra-shallow subwoofer featuring two 6.5 in transducers with optimized laminar vents to maximize LF efficiency and omnidirectional pattern - deployed in walls, in ceilings, on walls or on ceilings.

## Description

Install-specific ultra-shallow passive subwoofer with two direct radiating 6.5 in transducers mounted in a bass-reflex tuned enclosure with laminar vents for increased LF efficiency. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The transducer front is protected by a coated steel grill and an acoustically transparent fabric. Dedicated accessories allow for various mounting options: on-wall, on-ceiling, in-wall and in-ceiling. The inserts and the connectors are all gathered on a single enclosure panel. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage:

- Enclosure directivity: standard (omnidirectional)

Transducers:

- LF transducers: 2 x 6.5" neodymium cone driver

Acoustics:

- Nominal impedance: 4 Ω

- Usable bandwidth:

  - Preset 1: 29 - 107 Hz (-10 dB), 32 – 90 Hz (-6 dB), 34 – 75 Hz (-3 dB)

  - Preset 2: 29 - 150 Hz (-10 dB), 31 – 120 Hz (-6 dB), 35 - 100 Hz (-3 dB)

  - Preset 3: 32 - 300 Hz (-10 dB), 35 - 250 Hz (-6 dB), 40 - 200 Hz (-3 dB)

- RMS power handling (calculated using the mean impedance measured on the usable bandwidth): 85 W

- Maximum SPL (peak level at 1 m under half space conditions using pink noise with crest factor 4):

  - Preset 1: 112 dB

  - Preset 2: 113 dB

  - Preset 3: 117 dB

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 360 mm, 532 mm, 99 mm / 14.2 in, 20.9 in, 3.9 in

- Weight (net): 9 kg / 20 lb

- Connectors: 1 × 4-point terminal block with push-in connection

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: SB6i

Manufacturer: L-Acoustics