SOKa - ULTRA-SHALLOW colinear source

Medium throw loudspeaker: 35 m

Single element deployment

Line source directivity - Narrow vertically, wide horizontally

# Soka

## Short description

2-way ultra-shallow enclosure with nine 3.5 in neodymium drivers and three 1 in diaphragm compression drivers coupled to a waveguide to achieve a coherent wavefront with a 140° horizontal by 26° vertical coverage.

## Description

2-way passive enclosure with nine 3.5 in neodymium drivers and three 1 in diaphragm compression drivers coupled to a cylindrical waveguide with 140° of horizontal directivity and a tightly controlled 26° of vertical directivity. The enclosure cabinet is constructed of premium grade Baltic birch plywood with an ultra-shallow form for versatile integration. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. Dedicated accessories allow for on-wall and in-wall deployments. The inserts and the connectors are all gathered on a single enclosure panel. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Vertical directivity: +5°/-21°

- Horizontal directivity: 140°

Transducers:

- MF transducer: 9 x 3.5" neodymium cone drivers

- HF transducer: 3 x 1" diaphragm compression drivers

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth:

  - Preset 1: 60 – 20k Hz (-10 dB), 70 – 20k Hz (-6 dB), 100 – 20k Hz (-3 dB)

  - Preset 2: 100 - 20k Hz (-10 dB), 115 – 20k Hz (-6 dB), 135 - 20k Hz (-3 dB)

  - Preset 3: 200 - 20k Hz (-10 dB), 250 - 20k Hz (-6 dB), 300 - 20k Hz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 133 W

- Maximum SPL (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4):

  - Preset 1: 124 dB

  - Preset 2: 130 dB

  - Preset 3: 133 dB

Physical data:

- Acoustical load:

  - LF: closed enclosures

  - HF: cylindrical waveguide, laminar vents

- Cabinet: premium grade Baltic birch plywood

- Dimensions (W, H, D): 99 mm, 1065 mm, 99 mm / 3.9 in, 41.9 in, 3.9 in

- Weight (net): 9.4 kg / 20.3 lb

- Connectors: 1 × 4-point terminal block with push-in connection

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: Soka

Manufacturer: L-Acoustics