X6i - Short throw point source

Xi Series

Short throw: 15 m

Single element deployment (horizontal or vertical)

Axisymmetric directivity

# X6i

## Short description

2-way passive enclosure with 6.5 in / 1.5 in speaker producing an axisymmetric point source directivity of 90 degrees with consistent off-axis response.

## Description

2-way passive enclosure with a premium 1.5 in diaphragm compression driver coaxially loaded by a 6.5 in low-mid frequency transducer mounted in a bass-reflex cabinet. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure provides two angle settings of 35 and 55 degrees for stage monitoring. Dedicated accessories ensures wall-mount, ceiling-mount and flown configurations in vertical and horizontal orientations. The enclosure integrates eight M6 threaded inserts for external rigging elements. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Nominal directivity: 90° axisymmetric

- Monitoring angle: 35° / 55°

Transducers:

- LF transducer: 6.5" cone driver

- HF transducer: 1.5" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth:

  - Preset 1: 69 Hz - 20 kHz (-10 dB), 76 Hz - 20 kHz (-6 dB), 84 Hz - 20 kHz (-3 dB)

  - Preset 2: 54 Hz - 20 kHz (-10 dB), 60 Hz - 20 kHz (-6 dB), 67 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 83 W

- Maximum SPL: (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

  - Preset 1: 123 dB

  - Preset 2: 117 dB

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 187 mm, 362 mm, 170 mm / 7.4 in, 14.2 in, 6.7 in

- Weight (net): 6.3 kg / 14 lb

- Connectors: 	IN: 1 × 2-point terminal block + LINK: 1 × 2-point terminal block

- IP: IP55

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: X6i

Manufacturer: L-Acoustics